EEE 306

Experiment 5

Study of BUET Power Plant and Sub-station

Objective

The objective of this study is to get an exposure to a realistic small power plant and LT substation.

Introduction

BUET has a small (2 MW) power plant and its own distribution network and sub-stations to meet its electrical energy demand. The power plant is located at West Palasy campus of BUET and uses gas as its primary source of energy. The power plant supplies electricity to three sub-stations through under ground cable at 11 kV. The sub-stations distributes to each building, pump, street lighting at 230 V.

The peak demand of BUET campus at present is 1750 kW. The power plant supplies electrical energy to BUET campus from early morning till mid-night and from mid-night till early morning electrical energy is taken from the DESA distribution system. The changeover is done automatically by Automatic Transfer Switches (ATS) located at each sub-station.

Figure 1 shows in block diagram the layout for the BUET electricity generation and distribution system.

BUET Power Plant

ATS

ATS

ATS

ATS

ATS

Main SS (Panel 1)

Main SS (Panel 1)

Main SS (Panel 1)

NI Hall SS

Rashid Hall SS

11 kV Underground cable

DESA 11 kV Feeder

Figure 1. Block diagram of BUET power generation and distribution system

Power Plant

The BUET power plant consists of two 1 MW gas turbine generators and very soon another 2 MW generator will be incorporated in the system. The plant is fully automated with control and monitoring panels. It has protection system for generator –overload and –underload, generator short circuit, high radiator temperature, low gas pressure etc. The plant has noise suppressor to reduce noise.

Sub-station

All BUET substations consists of mainly the following components:

- Step down transformer

- Incoming and outgoing feeders

- HT and LT panel

- Circuit breaker

- Automatic Transfer Switch

- Relays

- Metering instruments (CT, PT and meter)

Record the following readings

1. Record all incoming and outgoing feeder data for the power plant and Panel-3 at the main substation in a table as shown below.

Table for feeder data record

| Date and Time | Feeder name | Incoming/ Outgoing | Voltage, V | Current, A | Power, kW | pf |
| --- | --- | --- | --- | --- | --- | --- |

2. Record the ratings of transformer, ATS, LT panel, PFI, circuit breaker at the power plant and at the main sub-station and at Rashid Hall sub-station.

Report

- Calculate the total current and power from the outgoing feeders and compare it with the incoming feeder readings.

- Draw the complete system of the BUET power plant using block diagram.

- Describe briefly the power plant protection system.