Specifications of this 4-bit PC

- 4-bit RAM

- 4-bit Program counter

- Capable to execute upto 16 instruction in a single program.

- RAM is able to keep upto 64 data which is maximum possible for 16 instructions this PC does. Address of RAM is 8 bit and saved in a instruction ROM.

- MAR is increasable; used in address related tasks.

- Dual BUS system optimizes time operations.

- Sophisticated auto data loading system loads RAM on beginning of eache execution in a speed of 16Hz. A LED is used in main circuit to show when this loading is going on.

- 9 T-state used.

- Variable execution cycle is used; which definitely made initial designing complicated a little, but very much efficient in memory consumption. It took 57 memory spaces in control ROMs; where if it was with constant execution cycle, it would take 114 memory spaces. So the variable execution cycles made it 50% less memory consumption.

- 2 temporary registers are used.

- “Clear” clears all the registers and flags & resets the counters.

- Shows output, input, BUS data, Program counter value, register A & B, T-state & executing instruction OP code in 7segment display.

- Instruction ROM contains address of all instruction in RAM. JZ address & CALL address indicates this instruction ROM. So, coding is easier. Any value of JZ (0 to F) will lead to an instruction.

- Whenever RAM sends data to BUS, it sends the same 4 bit data to both the BUSes and no other elements sends data then. Any component attached to the BUS can load this data as needed.

- Design is categorized with sub circuits.

- Colored design indicates portion of circuit used for any specified job. As-

In main circuit-

Yellow	-	Clock

Teal		-	Control sequence

Light Green -	Clear

Light Blue	-	Flags circuitry

In Control sub-circuit-

Purple	-	NOP condition

Orange	-	Zero Flag checking

MAR-

Yellow 	-	Stack pointer related circuit

RAM-

Brown	-	BUS to RAM writing ckt while loading

Light blue	-	Auto data loading to RAM system

Light green	-	Controlling ckt of completing data

loading to RAM and start execution.